Assignment 1

Total: 10 marks (will be converted into 5)

This assignment must be hand-written. Show ALL steps in ALL questions.

Read the questions carefully. Submit the graded part only.

Graded

- Convert the following numbers: [2 marks]

  - (ABC.14)14 = (?)7

  - (110101110111.11101010101111)2 = (?)16

[for the infinite fractional part, just do 4-5 steps and use dots for the rest.]

- Divide (513231)6 by (335)6 and state the result and reminder. You must do the calculation in base 6.  [2 marks]

- Convert the following streams of bit into decimals: [2 marks]

(11101011011101111010)Excess5

- Subtract 980 from 450 in 12 bits using 2’s complement number system and justify whether there is an overflow or not. If no overflow, then show the answer to decimal. [2 marks]

- Subtract 1450 from (-1980) in 12 bits using 1’s complement number system and justify whether there is an overflow or not. If no overflow, then show the answer to decimal. [2 marks]

The following part is given for your practice.

Ungraded

- Convert the following decimal number to equivalent binary numbers:

(a) (4693.23450)10

(b) (4976.250363)10

[for the infinite fractional part, just do 4-5 steps and use dots for the rest.]

- Convert the following base 9 number to equivalent base 5 numbers:

(a) (8712)9

(b) (234.256)9

- Convert the following binary numbers to equivalent hexadecimal numbers:

(a) (101010101010)2

(b) (110101110111.11101010101111)2

- Convert the following binary numbers to equivalent octal numbers:

(a) (101010101010)2

(b) (110101110111.11101010101111)2

- Perform the following base conversions

(a) (38A)13 = (?)7

(b) (10110111)7 = (?)4

(c) (0011)BCD = (?)5

(d) (0011)10 = (?)Excess3

(e) (110 0011)10 = (?)Excess5

- Perform addition, subtraction, multiplication and division for the pair of following base-8  numbers. Verify your results by converting the problem into decimal.

- 513

- 335

7. Perform addition, subtraction, multiplication and division for the pair of following base-6  numbers. Verify your results by converting the problem into decimal.

(a) 214

(b) 115

8. (010010101011111)2s = (?)10

9. (101010101000011)2s = (?)10

10. (010010101011111)1s = (?)10

11. (101010101000011)1s = (?)10

12. Subtract 13 from 27 in 7 bits using 2’s complement number system and justify whether there is an overflow or not.

13. Subtract 45 from 98 in 12 bits using 2’s complement number system and justify whether there is an overflow or not.

14. Add 13 with 27 in 6 bits using 2’s complement number system and justify whether there is an overflow or not.

15. Perform the following arithmetic operations using 13-bit two’s complement and one’s complement systems. State if there is an overflow in each case.

(a) 91- 499

(b) 379 + 98

16. You are a computer engineer and you want to buy two 8 GB DDR4 RAMs. Each  RAM costs (1C2)16 dollars. You also want to buy a graphics card RTX4070Ti which costs  (10010110000)2 dollars. However, you don’t have that much money with you and you are afraid to ask your parents about it. Suddenly, one of your generous friends agreed to give you the money you need. He decided to give you (4064)8  dollars. How much will you have left after buying those components?  [Show the answer in decimal]